Milestone 2 Software Engineering Enhancement

Joshua M. Collins

SNHU

CS-499, Neil Kalinowski

7/26/2026

The artifact chosen for the Software Engineering and Design enhancement was the temperature sensor project initially created in Python for CS-350. This project was based around an embedded system, specifically, a basic state machine. There were 3 states, off, heat, and cool. These state controlled LED, LCD, button functionality, and took input from an AHT20 sensor. For example, in the heat state, if the sensor input was lower than the setpoint, a variable initialized at 72 and incremented and decremented with buttons, the red LED would undergo pulse-width modulation and appear to fade in and out. At or above the set point, the LED would be solid. The LCD would also display alternating text on line 2, indicating the current mode, off, heat, or cool, as well as the set point, before reverting to just current reading and setpoint.

This artifact was included as it provided the opportunity to translate concepts found in a piece of software implemented in an interpreted language, to a compiled one. The original language, Python, provides ample opportunity for this. The Python language is one of the more accessible programming languages, and thus, has found wide adoption in embedded prototyping. This means that highly abstracted libraries are widely available for peripherals that interface with embedded-style communication protocols. C++ is a less accessible language, has fewer libraries available for this control, and the libraries it does have are more abstract. For example, the python project included libraries that handled the detection of rising edges for buttons and abstracted it away as an object with a when pressed function. That had to be manually configured with libgpiod in C++. The LCD 4-pin configuration also required manual configuration. Although the python version did require set up of pin numbers and names, the “bitbanging” was abstracted away. Initially, LCDProc was going to be used for this, but after consideration, the manual approach seemed more pragmatic than running a seperate daemon.

For this enhancement, my initially selected course outcome was, “Design and evaluate computing solutions that solve a given problem using algorithmic principles and computer science practices and standards appropriate to its solution while managing the trade-offs involved in design choices.”, I believe I have made strides towards this, yet there is quite a lot of room for improvement. I have not compared the resource usage of the C++ port to the initial Python project, but I know in some cases I have used types and logic that are less than optimal for the purpose. Given the program runs a Raspberry Pi 4 with Ubuntu Server, this is not currently a problem. The program will have no way of taking up all its memory, but if ported to production on a board with limited resources, it is possible. Regardless, the porting of this program shows the ability to understand the algorithmic principles of a program, determine the limits of the platform it is built on, and reconstruct it on a more solid foundation. While not robust, some error handling, e.g. the i2c file checks, exist within the code. This slightly widens the scope toward outcome 5. I plan to go over this enhancement during the next week while I work on the next enhancement to add additional error handling and prevent any overflows that may be possible in existing code.

One of the first challenges I faced was getting the LCD to display. My assumption was that this would be the most difficult portion of the hardware to get enabled, so I began with it and looked for ways to make it simpler. LCDProc initially seemed like it would be helpful. After wasting time attempting to configure it, and realizing that it would take some time to configure it, and require a concurrent background process to operate, I decided to look into the bitbanging approach. While still complex, I found it more approachable than I initially thought. Given the use of libgpiod throughout the project, I decided to switch to the manual approach using that library. Libgpiod brought its own challenges as well. Attempting to understand how to use the library, I looked for already created projects. A lot of these were older projects created on version 1 of libgpiod, which is now deprecated. I spent some amount of time trying to get the project to work based on version 1 projects. Obviously, this entire period was largely a waste. This taught me to go directly to the source for documentation and current information, as the libgpiod repository contains examples of its use.